REDUCED PROTOCOL DESCRIPTION                                 Page: 1

/ \

| R |

SSS    AAAA   III   AAAA  \ /      SSS   BBB    U  U    SSS

S      A   A    I   A   A          S      B  B   U  U   S

SSS   AAAAA    I   AAAAA    ---    SSS   BBB    U  U    SSS

S  A   A    I   A   A              S  B  B   U  U       S

SSS   A   A   III  A   A           SSS   BBB     UUU    SSS

REDUCED PROTOCOL DESCRIPTION

+============================+

CREATION  :   DD-EPFW6-025

+--------------------------+

G.Kershaw   15/11/91   Extracted from the Requirements

Specification 1 (E-PFW6-001)

G.Kershaw   16/07/92   Corrected example of tgm.

G.Kershaw   15/4/93    Included RV telegram

G.Kershaw   23/11/93   Included R/W DB, Read block addr

and Read block size telegrams

Th.Friedli  12/04/94   Included 'Read Hangup Timeout'

telegram for 'MODEMS+'.

1. INTRODUCTION

------------

1.1. Purpose

-------

The SAIA-BUS Protocol is a protocol which allows the exchange of

information across a single-client multiple-server network. The

protocol has the same functionality as the SAIA-P8 protocol but is

much simpler and so presents less loading on the processor and is

musch faster.

REDUCED PROTOCOL DESCRIPTION                                 Page: 2

TABLE OF CONTENTS

=================

1. INTRODUCTION ......................................................1

1.1. Purpose .........................................................1

2. OSI LAYER OVERVIEW ................................................3

2.1. Protocol Structure ..............................................3

3. APPLICATION LAYER .................................................4

3.1. General Description .............................................4

3.1.1. Variable Access Service .......................................4

3.1.2. Status Service ................................................4

3.1.3. Program Management Service and Program Commissioning Service ..4

3.2. Media Format ....................................................5

3.2.1. Integer Format ................................................5

3.2.2. Floating-Point Format .........................................5

3.2.3. Binary Format .................................................6

3.3. Application Protocol ............................................6

3.3.1. Write & control procedure: (as seen at the Application Layer) .6

3.3.2. Read procedure: (as seen at the Application Layer) ............6

4. PRESENTATION LAYER ................................................7

4.1. Syntax Representation ...........................................7

4.1.1. Example .......................................................7

4.1.2. General Definitions ...........................................8

4.2. Overview of all SBUS telegrams (Reduced & Full Protocol) ........9

4.3. Variable Access Service Telegrams ...............................10

4.3.1. Read Telegrams ................................................10

4.3.2. Write Telegrams ...............................................11

4.4. Status Service Telegrams ........................................11

5. EXTENSION FOR DATA BLOCKS .........................................12

6. EXTENSION FOR MODEMS+ .............................................13

7. NETWORK LAYER .....................................................14

7.1. Network Layer Service ...........................................14

7.1.1. The structure of a typical telegram ...........................14

8. DATA LINK LAYER ...................................................15

8.1. Upper Sub-layer .................................................15

8.1.1. Transmission of a command telegram ............................15

8.1.2. Invocation of response messages ...............................16

8.1.3. Half-Duplex Protocol ..........................................17

8.2. Lower Sub-layer .................................................17

9. PHYSICAL LAYER ....................................................18

REDUCED PROTOCOL DESCRIPTION                                 Page: 3

2. OSI LAYER OVERVIEW

------------------

2.1. Protocol Structure

------------------

The following diagram shows the implementation of the layers in

the SAIA-BUS protocol.

OSI model applied to SAIA-BUS:

������������������������������������������������Ŀ

�                    �                           �

� Application Layer  � SAIA-BUS (Reduced & Full) �

�                    � Protocols.                �

��������������������Ĵ                           �

�                    �                           �

� Presentation Layer � Telegrams 0 -> 255        �

�                    �                           �

��������������������Ĵ                           �

�                    �                           �

� Session Layer      � Not Used                  �

�                    �                           �

��������������������Ĵ                           �

�                    �                           �

� Transport Layer    � Not Used                  �

�                    �                           �

��������������������Ĵ                           �

�                    �                           �

� Network Layer      � Forced parity mechanism   �

�                    �                           �

��������������������Ĵ                           �

�                    �                           �

� Data Link Layer    �                           �

� ��������������-    �                           �

� Upper sub-level    � ACK/NAK/CR mechanism      �

� Lower sub-level    � byte synchronisation +    �

�                    � CRC-16 Error checking     �

��������������������Ĵ                           �

�                    �                           �

� Physical Layer     � RS485,RS232,20Ma CL etc   �

�                    �                           �

��������������������������������������������������

OSI - Open Systems Interface

S-BUS can run at the following speeds :-

110,150,300,600,1200,2400,4800,9600,19200,38400 baud

REDUCED PROTOCOL DESCRIPTION                                 Page: 4

3. APPLICATION LAYER

-----------------

3.1. General Description

-------------------

There will be two functional layers to the SBUS which are described here.

�����������������������������������������Ŀ

�     Program Commissioning Service       �

�                  &                      �

�     Program Management service          �

�      �������������������������Ŀ        �

�      � Status service          �        �

�      �          &              �        �

�      � Variable access service �        �

�      ���������������������������        �

�������������������������������������������

3.1.1. Variable Access Service

-----------------------

The following variables can be accessed by the SAIA-BUS protocol.

�����������������������������������������������������������Ŀ

�  Code  � Type                  � Format                   �

�����������������������������������������������������������Ĵ

�  R     � Register              � Integer / Floating point �

�  C     � Counter               � Positive integer         �

�  T     � Timer                 � Positive integer         �

�  F     � Flag                  � Binary                   �

�  I     � Input                 � Binary                   �

�  O     � Output                � Binary                   �

�  K     � The real time clock   � Special format           �

�  D     � The display register  � Any format               �

�������������������������������������������������������������

3.1.2. Status Service

--------------

The status of the PCD may be requested by the Client.  The following

states are allowed for each Processor in the PCD:

S       Stopped (by the debugger)

H       Halted  (by different reasons)

R       Running

C       Conditional running (Waiting for breakpoints)

D       Disconnected (Processor does not exist)

3.1.3. Program Management Service and Program Commissioning Service

------------------------------------------------------------

These services go to make up the FULL S-BUS protocol such as is used in

the SAIA PG3 DEBUGGER (SBUG). Full details of these services are beyond

the scope of this document.

REDUCED PROTOCOL DESCRIPTION                                 Page: 5

3.2. Media Format

------------

3.2.1. Integer Format

--------------

The integer format is based on 32 Bits with the following format:

Byte

|      0        |      1        |      2        |      3        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|s|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|x|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

31                                                             0

where     x - 31 bit value

s - sign

The sign bit is a zero for a positive value, and a one for a

negative value.

The range allowed by this format is as follows:

DECIMAL

2'147'483'647       >   VALUE    >  -2'147'483'648

BINARY (HEXADECIMAL)

7FFF'FFFF            >   VALUE    >   8000'0000

3.2.2. Floating-Point Format

---------------------

The floating point format is based on 32 Bits with the following

format:

Byte number

|      0        |      1        |      2        |      3        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|m|s|e|e|e|e|e|e|e|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

31                                                             0

where   m  - 24-bit mantissa

s  - sign of the number

e  - 7-bit exponent in excess 64 notation

The mantissa is considered to be a binary fixed point fraction and

except for zero it is always normalized (has a one bit in its highest

position). Thus, it represents a value of less than one but greater

than or equal to one-half. The sign bit is a zero for a positive

value, and a one for a negative value.

REDUCED PROTOCOL DESCRIPTION                                 Page: 6

The exponent is the power of two needed to correctly position the

mantissa to reflect the number's true arithmetic value. It is held

in excess 64 notation which means that the two's complement values

are adjusted upward by 64, thus changing 40H (-64) - 3FH (+63)

to 00H - 7FH. This facilitates comparisions among

floating-point values.

The value of zero is defined as all 32 bits being zeroes. The sign,

exponent, and mantissa are entirely cleared. Thus, zeroes are

always treated as positive.

The range allowed by this format is as follows:

DECIMAL                18                                 -20

9.22337177 * 10    >  +VALUE    >  5.42101070 * 10

18                                 -20

-9.22337177 * 10    >  -VALUE    > -2.71050535 * 10

BINARY (HEXADECIMAL)

63                                   -63

.FFFFFF  *  2       >  +VALUE    >   .FFFFFF  *  2

63                                   -63

-.FFFFFF  *  2       >  -VALUE    >  -.FFFFFF  *  2

3.2.3. Binary Format

-------------

Binary values are represented normally by one bit 0 or 1, up to 8

binary values ( flags,inputs,outputs ) can be stored in 1 byte.

3.3. Application Protocol

--------------------

Two different protocol procedures between a Client and a

Server are used.

3.3.1. Write & control procedure: (as seen at the Application Layer)

-------------------------------------------------------------

CLIENT                                            SERVER

�                                                �

��������������<write request>�������������������>� write or control

�                                                � action

The Client does not expect any response.

3.3.2. Read procedure: (as seen at the Application Layer)

--------------------------------------------------

CLIENT                                           SERVER

�                                                �

��������������<read request>��������������������>� read action

�                                                �

�<������������<read response>�������������������Ĵ

�                                                �

The Client expects a read response

REDUCED PROTOCOL DESCRIPTION                                 Page: 7

4. PRESENTATION LAYER

------------------

All the telegrams which are used by the SAIA-BUS Protocol are described

in this layer.

Most telegrams are of a fixed length and so there is no requirement

for a special end of telegram character. Those telegrams which are

not of fixed length have a count byte immediately following the

command code to indicate the length of the telegram. There is no

need for a count byte in the response telegram as the client will

already know the length of the telegram that he is expecting.

A telegram can have an absolute maximum length of 32 registers/timers/

counters or 128 flags/inputs/outputs when in run. Some special telegrams

can have more bytes than this but these telegrams cannot be used

when the CPU is in run. For instance, to optimise the downloading of

a program up to 64 program lines can be transferred at a time which

gives a maximum telegram length of 263 bytes.

4.1. Syntax Representation

---------------------

4.1.1. Example

-------

�� Function and name of telegram

�

�             �� Name of equivalent P8-Protocol telegram

�             �

�             �     �� Command code in hexadecimal

�             �     �

v             v     �

Read Input  'RI'    v

01 cmd ::=      <command_code> <count> <address-IOF>

^ sts ::=      {<bit-value>}+

� ^ ^

� � �

� � ���sts : Status telegram returned from the SERVER to the CLIENT

� �          ( The <read response> entity of the Application Layer )

� �

� �����cmd : Command telegram from the CLIENT to the SERVER

�            ( The <request> entity of the Application Layer )

�

�������01  :Command_code in decimal

{ <a> }+  represents :

- for read   (<r-count>) occurrences of the expression <a>

[ <a> ]   represents an optional parameter

REDUCED PROTOCOL DESCRIPTION                                 Page: 8

4.1.2. General Definitions

-------------------

<address-IOF>   ::= <2-byte>

<address-RTC>   ::= <2-byte>

<ascii-char>    ::= <1-byte>

<block-number>  ::= <2-byte>

<block-size>    ::= <3-byte>

<block-type>    ::= <1-byte> --> 07h <=> DB

<character>      := printing character code: 20hex -> 7Fhex

<clock-value>   ::= <week-year> <day-week> <year> <month> <day>

<hour> <minute> <second>

<cpu-number>    ::= <character>

<day>           ::= <1-byte> containing 2 BCD digits (01 - 31)

<day-week>      ::= <1-byte> containing 2 BCD digits (01 - 07)

<db-number>     ::= <2-byte>

<db-element>    ::= <2-byte>

<fio-byte>      ::= <1-byte> where each F/I/O is represented by a

bit in the byte.

<fio-count>     ::= <1-byte> to indicate the number of F/I/O's to

be written.

<hangup-timeout>::= <2-byte>

<hour>          ::= <1-byte> contianing 2 BCD digits (00 - 23)

<length>        ::= <1-byte>

<minute>        ::= <1-byte> containing 2 BCD digits (00 - 59)

<module-type>   ::= <5-character>

<month>         ::= <1-byte> containing 2 BCD digits (01 - 12)

<r-count>         ::= <1-byte>

Where the value indicates the number

of elements expected in the status telegram

returned to the client.

Wherever this byte is used the range of valid

values is indicated. This range is imposed

to limit the length of the telegram to a

maximum of 256 bytes.

<second>        ::= <1-byte> containing 2 BCD digits (00 - 59)

<status>        ::= <flags>

<version>       ::= <3-character>

REDUCED PROTOCOL DESCRIPTION                                 Page: 9

<w-count>       ::= <1-byte>

Where the value indicates the number

of bytes following the <w-count> location

until the end of the telegram. From this

the server can indirectly calculate the

number of elements to be written to.

Wherever this byte is used the range of valid

values is indicated. This range is imposed

to limit the length of the telegram to a

maximum of 256 bytes.

<week-year>     ::= <1-byte> contianing 2 BCD digits (01 - 52)

<year>          ::= <1-byte> containing 2 BCD digits (01 - 99)

<4-byte>        ::= <1-byte> <1-byte> <1-byte> <1-byte>

<2-byte>        ::= <1-byte> <1-byte>

<1-byte>        ::= 00 - FF hex, 0 - 255 decimal

4.2. Overview of all SBUS telegrams (Reduced & Full Protocol)

--------------------------------------------------------

DEC HEX Name of telegram

~~~ ~~~ ~~~~~~~~~~~~~~~~

REDUCED         00d 00h Read Counters

PROTOCOL :      01d 01h Read Display Register

~~~~~~~~~~      02d 02h Read Flags

(FW: Range-     03d 03h Read Inputs

checks         04d 04h Read Real Time Clock

included !)    05d 05h Read Outputs

06d 06h Read Registers

07d 07h Read Timers

10d 0Ah Write Counters

11d 0Bh Write Flags

12d 0Ch Write Real Time Clock

13d 0Dh Write Output

14d 0Eh Write Registers

15d 0Fh Write Timers

20d-27d 14h-1Bh Read PCD Status

32d 20h Read version

REDUCED PROTOCOL DESCRIPTION                                 Page: 10

4.3. Variable Access Service Telegrams

---------------------------------

4.3.1. Read Telegrams

--------------

Read Counter  'RC'

00 cmd  ::=  00 <r-count> <address-RTC>

sts  ::=  {<4-byte>}+

Where

Length of sts      ::= (<r-cnunt>+1) * <4-byte>

Range of <r-count> ::= 0 - 31

Read Display Register   'RD'

01 cmd  ::=  01

sts  ::=  <4-byte>

Read Flag   'RF'

02 cmd ::=   02 <r-count> <address-IOF>

sts ::=  {<fio-byte>}+

Where

Length of sts      ::= (<r-count>+1)/8 * <fio-byte>

Range of <r-count> ::= 0 - 127

Read Input   'RI'

03 cmd ::=   03 <r-count> <address-IOF>

sts ::=  {<fio-byte>}+

Where

Length of sts      ::= (<r-count>+1)/8 * <fio-byte>

Range of <r-count> ::= 0 - 127

Read Real Time Clock    'RK'

04 cmd  ::=  04

sts  ::=  <clock-value>

Read Output   'RO'

05 cmd ::=   05 <r-count> <address-IOF>

sts ::=  {<fio-byte>}+

Where

Length of sts      ::= (<r-count>+1)/8 * <fio-byte>

Range of <r-count> ::= 0 - 127

Read Register   'RR'

06 cmd  ::=  06 <r-count> <address-RTC>

sts  ::=    {<4-byte>}+

Where

Length of sts      ::= (<r-count>+1) * <4-byte>

Range of <r-count> ::= 0 - 31

Read Timer  'RT'

07 cmd  ::=  07 <r-count> <address-RTC>

sts  ::=    {<4-byte>}+

Where

Length of sts      ::= (<r-count>+1) * <4-byte>

Range of <r-count> ::= 0 - 31

REDUCED PROTOCOL DESCRIPTION                                 Page: 11

4.3.2. Write Telegrams

---------------

Write Counter   'WC'

10 cmd  ::=  0A  <w-count> <address-RTC> {<4-byte>}+

sts  ::=  empty

Where

Range of <w-count>     ::= 5 - 129

Write  Flag 'WF'

11 cmd  ::=  0B <w-count> <address-IOF> <fio-count> {<fio-byte>}+

sts  ::=  empty

Where

Range of <w-count>     ::= 2 - 17

Range of <fio-count>   ::= 0 - 127

Write Real Time Clock   'WK'

12 cmd  ::=  0C <clock-value>

sts  ::=  empty

Write  Output   'WO'

13 cmd  ::=  0D <w-count> <address-IOF> <fio-count> {<fio-byte>}+

sts  ::=  empty

Where

Range of <w-count>     ::= 2 - 17

Range of <fio-count>   ::= 0 - 127

Write Register  'WR'

14 cmd  ::=  0E  <w-count> <address-RTC> {<4-byte>}+

sts  ::=  empty

Where

Range of <w-count>     ::= 5 - 129

Write Timer  'WT'

15 cmd  ::=  0F  <w-count> <address-RTC> {<4-byte>}+

sts  ::=  empty

Where

Range of <w-count>     ::= 5 - 129

4.4. Status Service Telegrams

------------------------

Read PCD Status 'RS0'|'RS1'|'RS2'|'RS3'|'RS4'|'RS5'|'RS6'|'RS7'

cmd  ::=          14 | 15  | 16  | 17  | 18  | 19  | 1A  | 1B

20 - 27               RS0->6 cpu number ;  RS7 own

sts  ::=         'S' | 'H' | 'R' | 'C' | 'D'

Read System Program Version     'RV'

32 cmd  ::=  20

sts  ::=  <module-type> <version> <cpu-number>

Notes:      - 'empty' means : sending a 'NAK' or an 'ACK' as response.

~~~~~~      - For the BROADCAST mode there is NO response message !

REDUCED PROTOCOL DESCRIPTION                                 Page: 12

5. EXTENSION FOR DATA BLOCKS

-------------------------

Read Data Block         'RZ'

150 cmd  ::=  96 <r-count> <db-number> <db-element>

sts  ::= {<4-byte>}+

Where

Length of sts      ::= (<r-count>+1) * <4-byte>

Range of <r-count> ::= 0 - 31

Write Data Block         'WZ'

151 cmd  ::=  97 <w-count> <db-number> <db-element> {<4-byte>}+

sts  ::=  empty

Where

Range of <w-count> ::= 7 - 131

Read Block Addresses    'R1'

155 cmd  ::=  9B <r-count> <block-type> <block-number>

sts  ::=  {<address-prog>}+

Where

Length of sts           ::= (<r-count>+1) * <address-prog>

Range of <r-count>      ::= 0 - 31

Allowed block-types are ::= 7 - This is the only identifier

for DB's.

Read Block Sizes        'R2'

156 cmd  ::=  9C <r-count> <block-type> <block-number>

sts  ::=  {<block-size>}+

Where

Length of sts           ::= (<r-count>+1) * <block-size>

Range of <r-count>      ::= 0 - 31

Allowed block-types are ::= 7 - This is the only identifier

for DB's.

REDUCED PROTOCOL DESCRIPTION                                 Page: 13

6. EXTENSION FOR MODEMS+

---------------------

(See UNDO SBUS-PGU-PLM mechanism; project 'MODEMS+')

Read Hangup Timeout

145 cmd  ::=  91 <hangup-timeout>

sts  ::=  empty

Where <hangup-timeout> is defined as follows:

Unit: [Seconds], Range: 0..300 seconds, (MSByte first).

REDUCED PROTOCOL DESCRIPTION                                 Page: 14

7. NETWORK LAYER

-------------

7.1. Network Layer Service

---------------------

The network layer service is very simple and takes advantage of the

multidrop feature of the DUART used in the PCD family. This multidrop

mode eliminates the need for special start characters in each telegram.

This mode supports two different types of character, an address

character and a data character. The difference between the two is that

for an address character the parity bit is forced to 1 and for a data

character the parity bit is forced to 0.

A telegram consists of an address character followed by a number of

data characters targeted for a particular slave station. When any

address character is detected in the data stream the slave station

compares its address to the address character received before deciding

whether to receive the data characters in the telegram. Slave stations

which are not addressed continue monitoring the data stream for the

next address character.

The address 255 (dec) is to be reserved for broadcast messages. No

response is expected after transmission of a broadcast message.

The multidrop mode is a common feature which exists on a number of

very common microcontrollers/Duarts,for instance :-

Intel MCS-51 microcontroller family.

National Semiconductor INS8250 UART - The standard Uart for ALL IBM

and IBM compatible PC's.

Motorola 68681 DUART - The DUART used in the PCD series of PLCs.

7.1.1. The structure of a typical telegram

-----------------------------------

Take the telegram

Write Register 100 12345 Dec

to station 10 in the SAIA-BUS network. The telegram will look like

so :-

<0A><0E><05><00><64><00><00><30><39><CRC-16msbyte><CRC-16lsbyte>

�   �   �   �       �               �

�   �   �   �       �               �� Two bytes of CRC-16 code

�   �   �   �       �

�   �   �   �       �� Value 12345 (dec) in hexadecimal (4-byte)

�   �   �   �

�   �   �   �� Absolute address of register (2-byte)

�   �   �

�   �   �� count of number of bytes to follow in telegram

�   �      (not including the CRC bytes)

�   �

�   �� Command code

�

�� Address of station (Address Character).

REDUCED PROTOCOL DESCRIPTION                                 Page: 15

8. DATA LINK LAYER

---------------

The Data link layer can be divided into two sub-layers.

8.1. Upper Sub-layer

---------------

The Upper Sub-layer manages the point to point communication between

stations on the network. If a telegram is lost or corrupted then this

layer will manage the retransmission of this telegram. The

functionality of this level can be seen in the following diagrams.

There are two  response telegrams, these are NAK (the negative response)

and ACK (the positive response telegram).

When a telegram is received which is illegal, such as Change block

to a non-existent block or reception of a telegram which is not

supported by the services running in the server, then a NAK is returned.

For instance, if the server is running the reduced protocol services

and it receives a run command then a NAK will be returned. If a NAK

is received then the client immediately informs the upper layers

that it has transmitted an illegal telegram.

If a corrupted telegram is detected then there is no response and the

client will timeout up to three times before informing the upper

layers that there has been a transmission failure. The timeout

will be a function of the Baud rate.

8.1.1. Transmission of a command telegram

----------------------------------

This shows the successful transmission of a telegram.

CLIENT                                            SERVER

�                                                �

�����������<adr><cmd><CRC-16>�������������������>�

�                                                � Action

�<��������-ACK����������������������������������Ĵ

�                                                �

�����������<adr><cmd><CRC-16>����>next message invokes next action

�

REDUCED PROTOCOL DESCRIPTION                                 Page: 16

In the example below a corrupted telegram is received at the server

and the client receives no response. The telegram will then be

retransmitted after the client has timed out.

CLIENT                                            SERVER

�                                                � Error detected by

�����������<adr><cmd><CRC-16>�������������������>� the two bytes

�                                                � of CRC-16.

t_out ms                                            �

����������-<adr><cmd><CRC-16>�������������������>� Uncorrupted but

�                                                � illegal telegram.

�<��������-NAK����������������������������������Ĵ Action.

�                                                �

����������-<adr><cmd><CRC-16>������������������->� Legal telegram.

�                                                �

�<��������-ACK����������������������������������Ĵ Positive response.

�                                                �

If a message does not receive a response of any form after t_out ms

then it is repeated. A message is transmitted a total of three times,

ie there will be two retries.

The variable t_out will be dependent on the baud rate and will be

defined at a later date.

CLIENT                                            SERVER

�����������<adr><cmd><CRC-16>���������>?????     � Telegram lost.

t_out ms                                            �

�����������<adr><cmd><CRC-16>�������������������>� Retransmit

�                                                � telegram.

t_out ms ???<��� ACK�������������������������������Ĵ Response lost.

�                                                �

�����������<adr><cmd><CRC-16>������������������->� Repeat

�                                                � action.

�<��������-ACK����������������������������������Ĵ Response received.

�                                                �

8.1.2. Invocation of response messages

-------------------------------

Upon reception of the read telegram the server will transmit the

response directly. Any response which appears on the network must

be for the client, so there is no need for a special start of

telegram character or address character for the response.

This example shows the successful execution of a read response telegram

CLIENT                                            SERVER

�                                                �

�����������<adr><cmd><CRC-16>�������������������>� Reception of read

�                                                � telegram.

�<���������<sts><CRC-16>������������������������Ĵ Transmit response.

�                                                �

REDUCED PROTOCOL DESCRIPTION                                 Page: 17

A NAK reply to the read telegram will indicate to the client that the

telegram was illegal.

CLIENT                                            SERVER

�����������<adr><cmd><CRC-16>���������>?????     � Telegram lost.

�                                                �

t_out ms delay                                      �

�����������<adr><cmd><CRC-16>�������������������>� Transmission error

�                                                � detected by CRC-16.

t_out ms delay                                      �

�����������<adr><cmd><CRC-16>�������������������>� Uncorrupted but

�                                                � illegal telegram.

�<���������NAK����������������������������������Ĵ Action.

�                                                �

����������-<adr><cmd><CRC-16>�������������������>� Reception of valid

�                                                � new read telegram.

�<��������-<sts><CRC-16>������������������������Ĵ Transmit response.

�                                                �

8.1.3. Half-Duplex Protocol

--------------------

Only one station can be master in a network and so only a half duplex

protocol is supported. This means that there is never any danger of

deadlock of contending clients.

8.2. Lower Sub-layer

---------------

The main task of this layer is to manage the CRC-16 error checking

code. This type of error checking is used since this protocol uses no

form of parity checking on individual bytes.

The CRC-16 error checking algorithm uses the polynamial

X^16 + X^12 + X^5 + 1 = 1021 Hex

This is the standard CCITT CRC (Reference CCITT V-41). The procedure

for calculating the CRC-16 will be made available as a 'C' function.

The SAIA-BUS protocol will run at all standard baud rates, with

8 bits per character, forced parity and one stop bit. The baud rate

for the full protocol will be defined in the configure menu with a

default of 9600 baud.

The reduced protocol will be defined using the standard SASI

instruction.

REDUCED PROTOCOL DESCRIPTION                                 Page: 18

9. PHYSICAL LAYER

--------------

SAIA-BUS will run on all the types of communications ports of the PCD

family.

The SAIA S-BUS is designed principally to run over an RS485 Multidrop

network of one client and a maximum of 255 servers using an S-BUS repeater.

The SAIA S-BUS can also run over an RS-232 serial interface